GRAPHIC FILES GUIDE

Please follow these guidelines when submitting your project’s artworks:

#### LAYOUT FILES:

- PDF (PDF files are the preferred format for artworks. Please include the bleed area and save without template lines);

- Adobe InDesign (.indd – CS4 – 2019 versions. Please create the package with all fonts and linked images, or export to hi-res PDF without template lines);

- Adobe Illustrator (Please export to hi-res PDF or EPS without template lines);

- Adobe Photoshop (Please export to hi-res PDF without template lines);

- QuarkXPress (Please export to hi-res PDF without template lines)

#### PHOTOS OR IMAGES:

- PDF (.pdf)

- Photoshop (up to CS5.5 version) (.psd)

- TIFF (.tif)

- Photoshop EPS (.eps)

- JPEG (.jpg)

#### FONTS:

We accept PostScript, OpenType, or TrueType fonts.

- When providing the file’s package, please include all fonts used for your project.

#### RESOLUTION:

- All files must have a minimum resolution of 300 dpi (dots per inch). We gladly accept higher resolutions as well.

- Please avoid any image taken from the Internet. They are usually only 72 dpi and are not suitable for printing.

#### COLOR MODE:

- All files/images must be provided in CMYK color mode (cyan, magenta, yellow, black). (300dpi)

- Please provide all black and white files/images in greyscale color mode. (1200dpi)

#### BLEED:

The outer edge of the artwork is considered the “safety (or bleed) area” used for cutting. No important images or text should be placed within this area. Although 1/8 of an inch (0.125”) would the minimum size, we strongly recommend using ¼ of an inch (0.25”).  This is all provided in our templates, which can be download here. (qui si apre la pagina dei template)

#### SAFETY AREA:

During the finishing process there might be a slight displacement. Please keep all critical elements at least 1/8 of an inch (0.125”) from the final cut line (finished edge). We therefore suggest you allowing 1/4 of an inch (0.25”) for bleed.

#### VINYL LABEL ARTworks:

- Please submit all label’s artworks as a solid image, without central holes.

- All previously mentioned color and resolution standards apply to vinyl labels as well.

- Pantone colors are needed as reference for solid color inks on the vinyl’s label.

- All our different templates have guidelines showing its corresponding printing area. You can download here all vinyl label’s printing templates.

#### DELIVERY:

You can send your artwork files via email to INFO@PPMITALY.COM.